VISVESVARAYA TECHNOLOGICAL UNIVERSITY BELGAVI - 590018

A Mini project report on

“ARDUINO BASED ROBOTIC ARM

(PIC AND PLACE ROBOT)”

BY

PRAJVAL BADIGER

(USN: 4UB16EC029)

AND

ERANNA S. NAVALAGUNDA

(USN: 4UB16EC008)

UNDER THE GUIDANCE OF:

Mrs. GEETHA V

Prof, Dept. of Electronics and Communication

2017-2018

DEPARTMENT OF ELECTRONICS AND COMMUNICATION ENGINEERING

UBDT COLLEGE OF ENGINEERING, DAVANGERE-577004

UBDT COLLEGE OF ENGINEERING

DEPARTMENT OF ELECTRONICS AND COMMUNICATION

CERTIFICATE

This is to certify that Mini Project entitled “ARDUINO BASED ROBOTIC ARM” is a bonafide record of the Mini Project done by PRAJVAL BADIGER bearing USN: 4UB16EC029 under our guidance and supervision at the Department of Electronics and Communication Engineering, UBDT college of Engineering, Davangere, has completed the Mini project in the Fourth semester of Visveswaraya Technological  University,  Belagavi during the academic year 2017-2018.

----------------------------------------                                                                ----------------------------------------

Project guide                                                     Head of the department

Mrs. GEETHA V                                                       Dr. MANJANAIK
       Senior  Professor                                                                                          Professor

ACKNOWLEDGEMENT

I would like to thank all those who helped us to make this Mini Project a successful one.

First my special thanks to our guide Mrs. Geetha V, Senior Professor, Department of Electronics and Communication Engineering who guided to understand the topic and gave suggestions on improvement of this Mini Project and also for providing resources for the completion of this Mini project for her co-operation and encouragement at all of our approach.

I would like to thank Dr. Manjanaik HOD of Electronics and Communication, for providing us an platform for conducting this Mini Project.

Place: Davangere                                                                     PRAJVAL BADIGER

Date:	 26-5-2018                                                                           (4UB16EC029)

ABSTRACT

A Robotic Arm is a type of mechanical arm, usually programmable, with similar functions to a human arm; the arm may be the sum total of the mechanism or may be part of a more complex robot. The links of such a manipulator are connected by joints allowing either rotational motion (such as in an articulated robot) or translational (linear) displacement. The links of the manipulator can be considered to form a kinematic chain. The terminus of the kinematic chain of the manipulator is called the end effector and it is analogous to the human hand

The end effector, or robotic hand, can be designed to perform any desired task such as welding, gripping, spinning etc., depending on the application. For example, robot arms in automotive assembly lines perform a variety of tasks such as welding and parts rotation and placement during assembly. In some circumstances, close emulation of the human hand is desired, as in robots designed to conduct bomb disarmament and disposal.

In our case the Robotic Arm is going to pick and place any object from on palace to other within its radius.

CONTENTS

CHAPTER                                             PAGE NO.

- INTRODUCTION                                            6

- CIRCUIT AND WORKING                             8

- SOFTWARE PROGRAMMES                        10

- CONSTRUCTION                                            14

- PARTS LIST                                                     15

- CONCLUSION                                                 16

- REFERENCES                                                  16

CHAPTER 1

INTRODUCTION

An Arduino based Robotic arm is a robot which use micro servo motors for its operation and these motors and controlled by Arduino Uno. Let’s start knowing about the individual parts of the Robot.

1.Micro Servo Motors:

It is tiny and lightweight with high output power. This servo can rotate approximately 180 degrees (90 in each direction), and works just like the standard kinds but smaller. You can use any servo code, hardware or library to control these servos. It comes with a 3 horns (arms) and hardware.

Specifications

- Operating voltage: 4.8 V (~5V)

- Operating speed: 0.1 s/60 degree

- Stall torque: 1.8 kgf·cm

- Dead band width: 10 µs

- Temperature range: 0 ºC – 55 ºC

## How to connect SG90 micro servo motor to Arduino Uno?

### Hardware and Software Required

- SG90 Micro Servo motor

- Arudino Uno

- Arduino IDE(1.0.6V)

- Hardware connections

- The SG90 micro servo motor has 3 wire interface in which the connections are made as follows:

- Red wire-5V

- Brown wire-Ground

- Yellow wire-digital pin 9

2. Potentiometer  :

A potentiometer is a three-terminal resistor with a sliding or rotating contact that forms an adjustable voltage divider. If only two terminals are used, one end and the wiper, it acts as a variable resistor or rheostat.

The measuring instrument called a potentiometer is essentially a voltage divider used for measuring electric potential (voltage); the component is an implementation of the same principle, hence its name.

Potentiometers are commonly used to control electrical devices such as volume controls on audio equipment. Potentiometers operated by a mechanism can be used as position transducers, hence we can we potentiometer to control the position of the Robotic Arm.

3. Ardiuno Uno:

The Arduino Uno is the most common version of Arduino family. The Arduino Uno is a micro controller board based on the ATmega328.It has 14 digital input/output pins (of which 6 can be used as PWM outputs), 6 analog inputs, a 16 MHz ceramic resonator, a USB connection, a power jack, an ICSP header, and a reset button. The Arduino Uno is great choice for beginners. It contains everything needed to support the micro controller; simply connect it to a computer with a USB cable or power it with an AC-to-DC adapter or battery to get started. The Arduino Uno is a good choice for beginners since it is easy to start with.

## Technical Specifications

- Microcontroller- ATmega328

- Operating Voltage- 5V

- Input Voltage (recommended) - 7to12V

- Input Voltage(limit) - 6to20V

- Digital I/O Pins-14 (of which 6 provide PWM output)

- Analog Input Pins-6

- DC Current per I/O Pin-40 mA

- DC Current for 3.3V Pin-50 mA

- Flash Memory-32 KB (ATmega328) of which 0.5 KB used by boot loader

- SRAM-2 KB (ATmega328)

- EEPROM-1 KB (ATmega328)

- Clock Speed-16 MHz

CHAPTER 2

6

9

10

11

10

CIRCUIT AND WORKING

This robotic arm is made up done of three segment joined by 3 joints

The Ardiuno controls the robot by rotating individual Servo motors connected to each joint. Unlike dc motors, servo motors can be controlled with precise angles from 0 to 180 degrees . The angle of the servo motors are controlled with the help of potentiometer.

Potentiometers provide analog input to the ardiuno. These analog inputs and maped to angle of the servo motor hence the motors can be controlled using Arduino and potentiometers.Each joint of the Robotic Arm consist of a servo motor and movement of the arm are due to the angle change in the servo motor. The end effector is also controlled by potentiometer and can be changed for different applications

CHAPTER 3

SOFTWARE PROGRAMMES

The code for the project is written in Ardiuno IDE .Following is the program implemented for the Robotic Arm.

#include<Servo.h>

Servo servo0, servo1, servo2, servo3;

int analogout[4] = {0, 0, 0, 0};

int pin_button = 12;

int pin_button_state = 0;

int last_pin_button_state = 0;

int stor0[200];

int stor1[200];

int stor2[200];

int stor3[200];

int recording;

int sensorVal[4];

int storage_loc;

void setup()

{

servo0.attach(10);

servo1.attach(11);

servo2.attach(9);

servo3.attach(6);

pinMode(pin_button, INPUT);

pinMode(A0, INPUT);

pinMode(A1, INPUT);

pinMode(A2, INPUT);

pinMode(A3, INPUT);

}

void loop() // main loop

{

button();

mapval();

writeval();

record();

play();

}

void button() {

pin_button_state = digitalRead(pin_button);

if (pin_button_state != last_pin_button_state) {

if (pin_button_state == HIGH) {

delay(100);

recording++;

}

delay(50);

}

}

void mapval() {

if (recording == 0) {

sensorVal[0] = analogRead(A0);

sensorVal[1] = analogRead(A1);

sensorVal[2] = analogRead(A2);

sensorVal[3] = analogRead(A3);

analogout[0] = map(sensorVal[0], 0, 1023, 0, 180);

analogout[1] = map(sensorVal[1], 0, 1023, 0, 180);

analogout[2] = map(sensorVal[2], 0, 1023, 0, 180);

analogout[3] = map(sensorVal[3], 0, 1023, 0, 180);

}

}

void writeval() {

servo0.write(analogout[0]);

servo1.write(analogout[1]);

servo2.write(analogout[2]);

servo3.write(analogout[3]);

}

void record() {

if (recording == 1) {

sensorVal[0] = analogRead(A0);

sensorVal[1] = analogRead(A1);

sensorVal[2] = analogRead(A2);

sensorVal[3] = analogRead(A3);

analogout[0] = map(sensorVal[0], 0, 1023, 0, 180);

analogout[1] = map(sensorVal[1], 0, 1023, 0, 180);

analogout[2] = map(sensorVal[2], 0, 1023, 0, 180);

analogout[3] = map(sensorVal[3], 0, 1023, 0, 180);

writeval();

if (storage_loc < 180) {

stor0[storage_loc] = analogout[0];

stor1[storage_loc] = analogout[1];

stor2[storage_loc] = analogout[2];

stor3[storage_loc] = analogout[3];

delay(50);

storage_loc++;

}

}

}

void play() {

if (recording > 1) {

while (true) {

for (int i = 0; i < storage_loc; i++) {

servo0.write(stor0[i]);

servo1.write(stor1[i]);

servo2.write(stor2[i]);

servo3.write(stor3[i]);

delay(50);

}

}

}

}

***code ends***

CHAPTER 4

CONSTRUCTION

The whole body of the arm is made up of light weight wood making it eaiser to light weigth. Light weight wood is used since the servo motors have low torque and can’t handle more weight.

The wood is cut in proper dimensions for each segment and then groves are made in appropriate positions to fit servo motors in it.

The end effector is made of a zip tie and is fixed at the end of the top most segment. The end effector is such that it can hold any object, mimicking the hand hold gesture of humans.

The base is also made of wood on which the arm is mounted.

CHAPTER 5

PARTS LIST

- Micro Servo motors   SG90       - 4 no’s

- ARDUINO UNO                            - 1 no’s

- Potentiometers 10k                   - 4 no’s

- Battery  4.8v                                - 1 no’s

- Breadboard                                  - 1 no’s

- Push button                                 - 2 no’s

- Resistors 10k ohms                    - 1 no’s

- Connecting wires                              --

CONCLUSION

The International Federation of Robotics has predicted a worldwide increase in adoption of industrial robots and they estimated 1.7 million new robot installations in factories worldwide by 2020.Now a days robotic arms are used almost everywhere from industries to space robots . Due to its efficiency and accuracy  robotic arms are the most commonly used robot in automobile industries to paint, to tighten screws etc. The end effector can be changed for each task such as , a robotic arm that paints would have a paint spray its end effector.

Reference:

https://en.wikipedia.org/wiki/Robotic_arm

https://www.robotshop.com/letsmakerobots/micro-servo-robot